Novell NetWare ODI Server Driver Installation

���������������������������������������������

Installing the Ethernet drivers into a server computer outlined below.

Preparation

�����������

Before you start with the driver installation process to the server ,

make sure you have the following on hand:

1) PCMCIA Ethernet PC card

2) PCMCIA Driver diskette

Installation Procedure

����������������������

Installation of the  Ethernet driver to a server is  broken down into

two parts:

1) Hardware Installation (changes existing setting of the adapter)

2) Software Installation (copying appropriate driver to the server)

Hardware Install

����������������

Installing the PCMCIA into a server is the same with the installation

of workstation. You just setup the hardware configuration by software

for PCMCIA Card Enabler program, and record the final setting values.

Software Install

����������������

1) Login to the server as the "Supervisor" user.

2) Copy the server driver supplied  in the PCMCIA  Software diskette's

\NOVELL\SERVER\3.1X  subdirectory into  the  server's   SYS:SYSTEM

directory  for   Netware 3.1X   or  copy  the   \NOVELL\SERVER\4.X

directory into the server's SYS:SYSTEM directory for Netware 4.X.

3) Use  the LOAD command from the System Console to load the PCMSR.LAN

file into the server's memory. Normally, the driver would load from

SYS:SYSTEM directory of NetWare partition , but it could optionally

be  loaded from  a floppy disk or a directory on a DOS partition of

the server's hard disk instead.

A typical command line to load the server driver is:

LOAD PCMSR.LAN PORT=300, INT=3

The LOAD line options may be entered in any sequence and the commas

in between them are optional, as are most of the options themselves.

However ,  the PORT and INT options  MUST be included and they MUST

equal the physical hardware settings.

Following are the parameters that may be included in the LOAD line:

PORT : The Base I/O Address at which the PCMCIA hardware has been set.

A drive denominator may be added to the port address in order

to  specify  a  driver  location  other than  the  SYS:SYSTEM

directory of the NetWare Partition.  For example:

... PORT=300 ... <���� Load driver from SYS:SYSTEM (default)

INT  : This is the interrupt level (IRQ) that the adapter has been

configured to use.

4) You may  optionally wish to put the LOAD line in the STARTUP.NCF or

AUTOEXEC.NCF auto-load driver files so that the LOAD statement will

take effect when the server system is initialized.

Closing Notes

��������������������������������������������������

1) Appendix "E"  of the Novell NetWare 386 Installation Guide provides

more information about the usage of NET.CFG files.

2) Before you running any ODI drivers, you have to enable the PCMCIA PC

card by running ecard.exe or ecardcs.exe.